|  | KSHEMANKHARI CareCompanion |  |  |
| --- | --- | --- | --- |
| Introduction: Elderly individuals and patients often face difficulty managing their medications and monitoring their own health on a daily basis, which can lead to missed doses or unnoticed health issues. This also places a significant burden on caregivers, who must provide constant supervision. CareCompanion was developed to address this challenge. It is a service robot that supports elderly and patient users by identifying medicines, providing schedule reminders, and monitoring basic health parameters. Through this project, we aim to demonstrate how accessible robotic technology can improve daily care, promote safety, and reduce the workload placed on caregivers. |  | Method: CareCompanion was built based on IoT based microcontroller named ESP32. This is integrated with MAX30102 sensor for BMP, SPO2, Body temperature measurement, SIM800L GSM for Emergency Calling, HuskyLens for Medicine Detection, BN220 Micro GPS Module for tracking if the person is lost, MPU6050 Gyro for fall or accident detection and an OLED for notification and Measurement Updates, Buzzer for notification. This IoT system is based on Arduino IoT cloud for rapid prototyping and reliable deployment. |  |
| Data and Discussion: First, we approached with ESP Rainmaker platform for seamless integration but this created greater problem down the road and ESP32 was overwhelmed. Data taken from MAX30102 Sensor was 95% reliable, but was taking a bit longer than usual (55 seconds). Taking a much shorter approach. Medicine tag detection was 98% correct. Fall detection worked 80% of the time but there were no false signals, there must be increased intensity. |  | Conclusion: We tested CareCompanion with an elderly person and got good results. The heart rate monitor worked correctly and the buzzer was loud enough to hear. The person said they liked the medicine reminders; it helped them remember to take their pills on time. The GPS was useful so their family could check on them. Overall, she hardly noticed any discomfort, and it’s like a digital helper for her. CareCompanion did what it was supposed to do. |  |
| Reference: - https://pmc.ncbi.nlm.nih.gov/articles/PMC3234383/ - https://drkumo.com/causes-impacts-and-solutions-of-medication-non-compliance/ - https://cmed.com.bd/ - https://www.paroseal.co.uk/ - https://www.robotics.org.za/DFR0299 - https://www.allaboutcircuits.com/electronic-components/datasheet/MPU-6050--InvenSense/ - https://www.cyberdyne.jp/english/products/HAL/ |  |  |  |
| Recommendation: The stick could fall out of hand and trigger false alert; we are working on it. The heart rate sensor would take a long time to show the value, so we went to fix the algorithm to make the scanning faster with much lower sampling time. Alongside that, the GPS wasn’t performing up to the required specification, so we upgraded the old hardware into a much more compact and modern form factor, so that we could accommodate in a much smaller package. Now the entire thing is much easier to assemble with any sticks. |  |  |  |